C172/A N1908F Checklist

Preflight

Cockpit

Ignition Sw	Off

Control Lock	Remove

Master Sw	On

Fuel		Check Gauges

Beacon		Check on

Flaps		Extend

Master Sw	Off

Fuselage/Empennage

Baggage Door	Locked

Antennas	Secure

Controls	Free/Cables Secure

Hinges		Secure Pins/Free Moving

Trim Tab	Check

Tail Tie Down	Release

Right Wing

Flap/Aileron	Guides/Rollers/Security/Movement

Wing Tip	Lights/Nav & Strobe

Leading Edge	Smooth

Tie Down	Release

Brakes/Tires	Disc/Pad/Leaks/Inflation/Tread

Fuel		Check QTY/Strain/Sump

Nose Section

Oil		6.5-7.5qts

Fuel		Strain/Sump

Alt Belt	Check Tension/Condition

Air Intake	Clear-Check Filter

Wheel/Strut	Tread/Inflation/Position

Prop		Dents/Cracks/Nicks/Bent

Static Port	Clear

Left Wing

Fuel		Check QTY/Strain/Sump

Leading Edge	Smooth

Stall Horn	Check

Pitot Tube	Clear/Uncover

Tie Down	Release

Wing Tip	Lights/Nav & Strobe

Flap/Aileron	Guides/Rollers/Security/Movement

Brakes/Tires	Disc/Pad/Leaks/Inflation/Tread

C172/A N1908F Checklist

Preflight Complete

Before Starting

Seat/Belts	Adjust/Secure

Head Sets	Jack In

Fuel Selector	Both

Avionics Sw	Off

Circuit Brkrs	In

Head Sets	Jack In

Starting

Carb Heat	Off

Mixture	Rich

Primer		As Needed/In/Secure

Throttle	Off Idle ½”

Master Sw	On (beacon is on now)

Clear Area	Alert Call

Ignition	Start

Oil/Amps/Vac	Verify Nominal

Flaps		Retract

Avionics Sw	On

Transponder	Standby

Taxi

ATIS		Receive/Copy

Ground	Contact/Clear

Alt		Set

Brakes		Test

Wind Dir.	Verify with Sock

Controls	Position as req.

C172/A N1908F Checklist

Start/Taxi Complete

Before Takeoff

Park Brake		Set

Doors/Window	Closed/Secure

Flight Controls	Free Full range

Trim			Take Off Set

Instruments		Set Alt/Gyros/Etc.

Fuel Selector		Both

Mixture		Rich

Throttle		1700rpm

Mags			Check 100rpm drop

Carb Heat		Check 50-100rpm drop

Engine Instruments	Nominal

Throttle		1000rpm

Radio			Tower

Transponder		Alt

Park Brake		Release

Normal Flight Landing

Before Landing

ATIS			Receive/Copy

Tower			Contact/Clear

Seat/Belts		Adjust/Secure

Doors/Window	Closed/Secure

Fuel Selector		Both

Carb Heat		On

Mixture		Rich

Flaps			As req.

Approach		Stabilized